CNC Equipment Specifications

Mazak Machine Portfolio — Site Zero Configuration

PRELIMINARY

Aurelian Manufacturing AS  |  VDR 04  |  February 2026  |  Confidential

Source: Ravema AB Offers 25011042 / 25011043 / 25011044  |  Dated 2025-11-24

1. Summary

Site Zero is configured with five Mazak CNC machines in three types, covering multi-tasking, 5-axis milling, and turning with milling capability. All machines are sourced through Ravema AB (Nordic Mazak distributor) and configured for progressive automation toward lights-out operation.

Note: This configuration is preliminary and represents the estimated scope of equipment required to serve the preferred client base. Final machine selection and option packages will be confirmed during facility planning.

| Parameter | Specification |
| --- | --- |
| INTEGREX i-450HS×1500 | Multi-tasking turn/mill center — SmoothAi — 3 units |
| CV5-500 | 5-axis vertical machining center — SmoothX — 1 unit |
| QUICK TURN 250MSY | Turning center with milling + Y-axis + 2nd spindle — SmoothG — 1 unit |

Supporting equipment includes a dedicated metrology room (Wenzel CMM, Jenoptik Opticline), material preparation (bandsaw, storage), and facility systems (Kaeser compressed air, Absolent oil mist extraction).

2. INTEGREX i-450HS × 1500

Mazak INTEGREX i-450HS — representative image (source: Ravema AB)

Multi-tasking center combining turning and milling in a single setup (DONE IN ONE®). Two spindles, B-axis milling head, 74-tool chain magazine. Primary production machine at Site Zero.

Offer: 25011043  |  Qty: 3  |  Control: MAZATROL SmoothAi

Key Specifications

| Parameter | Specification |
| --- | --- |
| Axes | X, Y, Z, B, C1, C2 (index) |
| Max machining dia / length | Ø 670 mm / 1,500 mm |
| Main spindle | 12″ chuck, 37 kW, 3,300 RPM, Ø 112 mm bore |
| Second spindle | 10″ chuck, 26 kW, 4,000 RPM |
| Milling spindle | B-axis 240° travel, 12,000 RPM |
| Tool magazine | 74 tools — Capto C-6, chain magazine, ID chips |
| Coolant | 70 bar through-spindle, programmable 7 steps |
| Automation | Auto door, robot interface Ethernet/IP, MazaCheck |
| Probing | Renishaw RMP600QE + Inspection Plus |
| AI features | Smooth AI vibration control, energy monitoring |
| Chip handling | ConSep conveyor, right-side discharge |
| Footprint | 6,500 × 4,000 mm (machine) + 4,000 × 4,150 mm (automation) |

Preliminary Order Options

| Code | Description |
| --- | --- |
| A2271500U | Base machine — INTEGREX i-450HSx1500, SmoothAi |
| A227NC611 | Extra screen |
| A227SD140 | Foundation kit (anchor bolts and plates) |
| A227SI098 | C-axis spindle 2 — programmable 0.0001° |
| A227AT375 | 74-tool chain magazine — Capto C-6 |
| A227MM053 | Tool for mounting/dismounting ID chips |
| A227AS066 | Energy monitoring |
| A227CP253 | Smooth Tool Management |
| A227CP514 | Smooth AI milling spindle — vibration minimization |
| A227AS080 | Cutting force monitoring |
| A227MO084 | Portable pulse generator |
| A227PL083 | Status light tower (3-colour) |
| A227CS527 | 70 bar coolant, programmable 7 steps |
| A227CS808 | Prep for 70 bar through-spindle (40 μm filtration) |
| A227AS321 | Automatic door (1500 mm spec) |
| A227CS668 | Prep for oil mist separator (Losma GX1000) |
| A227MB277 | Renishaw RMP600QE probe |
| A227MB621 | MazaCheck |
| A227MB624 | Renishaw Inspection Plus software |
| A227MB650 | Reference sphere with magnetic base |
| A227GL015 | Robot interface — Ethernet/IP |
| A227CV350 | ConSep chip conveyor, right discharge |
| A227CV512 | Prep for ConCep chip conveyor |
| A227TE810 | Multi-tool function 24 positions |
| A227NO056 | Mazak API |
| A227NO059 | MT Connect |
| A227NO066 | Synchronized tapping rotary tools (G84.2) |
| A227NO116 | EIA/ISO simultaneous 5-axis |
| A227NO117 | Tool tip control 5-axis (G43.4) |
| A227NO127 | Engraving option |
| A227NO128 | Cutting point control 5-axis (G43.8) |
| A227NO134 | Simultaneous 5-axis spindle 2 |
| A227NO159 | Max program size 32 MB |
| A227NO442 | G00 Slope Constant |
| A227NO482 | Re-threading function |
| A227NO484 | Variable thread start position |
| A227NO507 | Coordinate rotation internal turning |
| A227NO531 | Turn-milling with Mazatrol |
| A227NO599 | Cylindrical and polar coordinates (G12.1 / G07.1) |
| A227YI360 | V-axis control |
| A227R1001 | ID chips for Capto holders (75 pcs) |
| A227PTekn | Production technical assistance — 5 days |
| A227DDP01 | DDP delivery to customer site |

3. CV5-500

Mazak CV5-500 — representative image (source: Ravema AB)

5-axis vertical machining center with bridge construction and dual-supported trunnion table. Roller Gear Cam drive in rotary axes eliminates backlash.

Offer: 25011044  |  Qty: 1  |  Control: MAZATROL SmoothX

Key Specifications

| Parameter | Specification |
| --- | --- |
| Axes | X, Y, Z, B (±110°), C (360° continuous) |
| Table diameter | Ø 500 mm |
| Spindle | 18,000 RPM, 18.5 kW, CAT-40 |
| B/C indexing | 0.0001° — Roller Gear Cam (zero backlash) |
| Tool magazine | 48 tools, CAT-40 |
| Coolant | 15 bar through-spindle and around spindle |
| Automation | Auto side-door, robot interface Ethernet/IP |
| Probing | Renishaw RMP-600 + RLP 40 + MazaCheck |
| Chip handling | Mayfran CleanSweep, bed and guard flushing |

Preliminary Order Options

| Code | Description |
| --- | --- |
| D34000000 | Base machine — CV5-500, SmoothX |
| D340CS089 | Prep for oil mist separator |
| D340CS213 | Through-spindle air blast (during rotation) |
| D340MM006 | Tool ID system (touchpanel) |
| D340SR395 | 18,000 RPM spindle, CAT-40 |
| D340AT050 | 48-tool magazine, CAT-40 |
| D340AS080 | Cutting force monitoring |
| D340MO060 | Portable pulse generator |
| D340PL082 | Status light tower (3-colour) |
| D340UK350 | Coolant package 4 — CleanSweep, 15 bar through + around spindle |
| D340PD296 | Automation package 1 — auto side-door, robot interface, table ports |
| D340UK358 | Measurement package 2 — RMP-600, RLP 40, MazaCheck |
| D340MO006 | T-slot table |
| D340NO056 | Mazak API |
| D340NO059 | MT Connect |
| D340NO091 | Synchronized tapping |
| D340NO116 | EIA/ISO simultaneous 5-axis |
| D340NO117 | Tool tip control 5-axis (G43.4) |
| D340NO123 | Radius compensation 5-axis |
| D340NO127 | Engraving option |
| D340NO157 | Max program size 8 MB |
| D340NO442 | G00 Slope Constant |
| D340PTekn | Production technical assistance — 4 days |
| D340DDP01 | DDP delivery to customer site |

4. QUICK TURN 250MSY 1000 mm

Mazak QUICK TURN 250MSY — representative image (source: Ravema AB)

Turning center with milling capability (M), second spindle (S), and Y-axis (Y). Handles complete parts in one setup. Direct-drive turret eliminates belt wear.

Offer: 25011042  |  Qty: 1  |  Control: MAZATROL SmoothG

Key Specifications

| Parameter | Specification |
| --- | --- |
| Axes | X, Y, Z, C1, C2 (index) |
| Max swing / machining dia | 673 mm / 375 mm |
| Max machining length | 1,000 mm between chucks |
| Main spindle | 10″ chuck, 26 kW, 4,000 RPM, Ø 91 mm bore |
| Second spindle | 6″ chuck, 11 kW, 6,000 RPM |
| Turret | 12-position direct-drive, rotary tools 7.5 kW / 6,000 RPM |
| Y-axis travel | ±52.5 mm |
| Coolant | 15 bar |
| Chip handling | Lamell conveyor (Crowther), side discharge |

Preliminary Order Options

| Code | Description |
| --- | --- |
| B7ZK1000U | Base machine — QT 250MSY 1000 mm, SmoothG |
| B7ZKCS176 | Through-turret air blast |
| B7ZKPD106 | Productivity package Crowther — bar feeder interface, auto part catcher, 15 bar coolant, chip conveyor |
| B7ZKSI095 | C-axis spindle 2 — programmable 0.0001° |
| B7ZKAS080 | Cutting force monitoring |
| B7ZKPL083 | Status light tower (3-colour) |
| B7ZKCS628 | Prep for oil mist separator G500 |
| B7ZKNO059 | MT Connect |
| B7ZKNO127 | Engraving option |
| B7ZKNO157 | Max program size 8 MB |
| B7ZKNO482 | Re-threading function |
| B7ZKNO602 | Cylindrical interpolation (G07.1) |
| B7ZKPTekn | Production technical assistance — 4 days |
| B7ZKDDP01 | DDP delivery to customer site |

5. Support Equipment

5.1 Metrology Room

| Parameter | Specification |
| --- | --- |
| CMM | Wenzel LH 87 Revo — parts up to Ø 350 mm, 1,500 mm length |
| Optical | Jenoptik Opticline — shaft measurement and contour analysis |
| Handheld | Micrometers, bore gauges, surface roughness testers, calipers |

5.2 Material Preparation

| Parameter | Specification |
| --- | --- |
| Automatic bandsaw | Max Ø 360 mm, programmable cut lengths |
| Manual bandsaw | Backup / secondary cuts |
| Storage | Raw material shelving with traceability |

5.3 Facility Systems

| Parameter | Specification |
| --- | --- |
| Compressed air | Kaeser compressor + Secotec refrigeration dryer |
| Oil mist extraction | Absolent system, ducted for 4 machines |
| Material handling | Forklift, 3.5 t capacity |

6. Automation Readiness

All machines are specified with automation-ready options supporting progressive rollout from manual operation to lights-out production.

| Parameter | Specification |
| --- | --- |
| Feature | INTEGREX (3) / CV5-500 (1) / QT 250MSY (1) |
| Robot interface | Ethernet/IP on all INTEGREX + CV5-500 |
| Auto door | INTEGREX (auto door) + CV5-500 (auto side-door) |
| In-process probing | Renishaw RMP600QE (IG) / RMP-600 (CV5) |
| Geometry check | MazaCheck on INTEGREX + CV5-500 |
| Tool management | Smooth Tool Management + ID chips (INTEGREX) |
| AI optimisation | Smooth AI vibration control (INTEGREX) |
| Connectivity | MT Connect + Mazak API on all machines |
| Bar feeding | Crowther bar magazine interface (QT 250MSY) |

Control platforms: SmoothAi (INTEGREX), SmoothX (CV5-500), SmoothG (QT). All share Mazatrol conversational programming and iCONNECT cloud monitoring.

7. Commercial Terms

Identical terms apply across all three Ravema offers.

| Parameter | Specification |
| --- | --- |
| Delivery | FCA Göteborg, upgraded to DDP customer site (Hølen) |
| Delivery time | To be agreed (subject to global supply chain) |
| Payment | 30% at order / 60% at delivery / 10% after approval — all 30 days net |
| Currency | Based on 1 EUR = 11.80 SEK, adjusted at payment date |
| Training | Mazatrol course at Ravema Värnamo (2 persons/machine) + on-site 8 hrs |
| Tech assistance | 4–5 days per machine, included |
| Warranty | 12 months regardless of operating hours |
| Warranty conditions | Preventive maintenance every 3,000 hrs, documented |
| General terms | NL 01 (Swedish standard delivery terms) |
| Non-proliferation | Relocation detector — locks if moved without Mazak authorisation |

8. Reference Documents

Supplier Offers

▸ 25011042 — Quick Turn 250MSY 1000 mm

▸ 25011043 — INTEGREX i-450HSx1500

▸ 25011044 — CV5-500

Machine Specs & Layouts

▸ Maskinspec_Quick_Turn_250MSY_1000_mm_2.pdf

▸ Maskinspec_Integrex_i450HSx1500_3.pdf

▸ Maskinspec_CV5500_8.pdf

▸ Layout and foundation drawings (QT, IG, CV5)

Support Equipment

▸ Secotec dryer, Kaeser compressor & air receiver prospectuses

▸ Investeringer_Maskiner_og_Utstyr.xlsx — equipment register

Related VDR Documents

▸ 04_CNC_Resale_Value_Analysis.pdf

▸ 04_Automation_Feasibility_Study.pdf